SOP

for Setting of Switch

|  | Created By | Reviewed By | Approved By |
| --- | --- | --- | --- |
| Name |  |  |  |
| Designation |  |  |  |
| Organization |  |  |  |
| Signature |  |  |  |
| Date |  |  |  |

# SOP for Setting of TP-Link Switch SG3210

## General Settings

- Kindly go to Virdi device Settings as mentioned above.

- Based on the network settings of the Virdi Device, kindly put up the gateway IP in the Google Chrome URL, and check if it opens the Switch Webpage.

- If no, kindly change IP address to the previous or next IP address as that of Virdi device, till the web page of the TP Link switch is opened.

- In SD WAN, LAN switch needs to have IP address for first time configuration for Godrej HO team to cross verify.

- This setting can be removed by BPCL from SD WAN, after intimation from Godrej team.

## User Settings

- Once Switch Web Page is opened, kindly Login into the switch using default credentials.

Login: admin

Password: admin

- Once logged in, there will be a menu popped up to change the default password.

- Kindly change the default Admin password to a strong password consisting of Uppercase and Lowercase alphabets, numbers and a special key.

- The password can also be changed later by entering System.

- Then to User Management

- Check on Password Modify, so that the three fields below will get enabled.

- Then enter the current password, new password and then confirm new password as per the Strong password policy. Then click on Save.

- The new password for login page will then get saved.

## Security Management

- To impose the restriction on no. of Admin users, kindly go to Security.

- Enable the Number Control.

- Enter the Number of Operators to 1, so that only one Admin is created and can login to the System Settings. Click on Apply.

- Also set the Session Timeout to be 5 Seconds. Click on Apply.

## Port Hardening

- Kindly enter L2 Features.

- In switching, click on Port Config

- Kindly Go to UNIT1

- Select the physical ports which are not connected by selecting the checkboxes.

- Kindly set the status as Disabled. Click on Apply.

- This will disable all the open physical ports which are unused.

## MAC ID Binding

- Click on Security.

- In Access Security, click on Access Control

- In Global Config, Enable the Access Control

- Select Control Mode to “MAC based”

- Click on Add and kindly select all the protocols other than TELNET.

- Then enter the respective MAC ID

- Repeat the above process for the MAC IDs of all the Virdi devices as well as PC connected to TP Link Switch.

- Click on Apply.

## HTTP Configuration

- Click on Security.

- In Access Security, click on HTTP Config

- Select HTTP Enable checkbox. Click Apply.

- Set Session Timeout to minimum of 5 minutes. Click Apply.

- Enable Number Control checkbox.

- Set Number of Admins to 1. Then click Apply.

## HTTPS Configuration

- Click on Security.

- In Access Security, click on HTTPS Config

- Select HTTPS Enable checkbox. Click Apply.

- In Session Config, set Session Timeout to minimum of 5 minutes. Click Apply.

## SSH Configuration

- Click on Security.

- In Access Security, click on SSH Config

- Select SSH Enable checkbox. Click Apply.

- Set Session Timeout to 360 seconds.

## DoS Protection

- Click on Security.

- Click on DoS Defend. Click Apply.

- Enable all the checkboxes in DoS Defend Config. Click Apply.